Source Code

/* USER CODE BEGIN Header */
/**
  ******************************************************************************
  * @file           : main.c
  * @brief          : DHT11 TrustZone-style TinyML anomaly detection prototype
  ******************************************************************************
  * Board:
  * - NUCLEO-U5A5ZJ-Q
  *
  * Connections:
  * - DHT11 VCC  -> 3.3V
  * - DHT11 GND  -> GND
  * - DHT11 DATA -> Arduino D7 / PF13
  *
  * LED:
  * - LD2 Blue -> PB7
  *
  * Current prototype stage:
  * - Single-project implementation
  * - TrustZone behaviour is logically simulated
  * - DHT11 sensor input is real
  * - TinyML inference is still simulated using a lightweight scoring function
  * - HSE/verification is simulated using a signature-like calculation
  *
  * LED meanings:
  * - Slow blink              = normal monitoring
  * - Fast blink burst         = verified anomaly alert
  * - 1 second ON/OFF blink    = blocked access, tamper, or sensor error
  *
  * UART:
  * - UART is initialized, but printf/transmit is not used because it previously
  *   caused blocking on this setup.
  ******************************************************************************
  */
/* USER CODE END Header */

#include "main.h"
#include <stdio.h>

/* Private variables ---------------------------------------------------------*/
UART_HandleTypeDef huart3;
PCD_HandleTypeDef hpcd_USB_OTG_HS;

/* DHT11 pin configuration */
#define DHT11_GPIO_PORT GPIOF
#define DHT11_GPIO_PIN  GPIO_PIN_13

/* Private function prototypes -----------------------------------------------*/
void SystemClock_Config(void);
static void SystemPower_Config(void);
static void MX_GPIO_Init(void);
static void MX_ICACHE_Init(void);
static void MX_USB_OTG_HS_PCD_Init(void);
static void MX_USART3_UART_Init(void);

/* -------------------------------------------------------------------------- */
/* PROTOTYPE TYPES                                                            */
/* -------------------------------------------------------------------------- */

typedef enum
{
    NODE_STATE_NORMAL = 0,
    NODE_STATE_ANOMALY = 1,
    NODE_STATE_VERIFIED_ALERT = 2,
    NODE_STATE_ACCESS_BLOCKED = 3,
    NODE_STATE_TAMPER_BLOCKED = 4,
    NODE_STATE_FEATURE_BLOCKED = 5,
    NODE_STATE_SENSOR_ERROR = 6
} NodeState_t;

typedef struct
{
    uint32_t temperature_c;
    uint32_t humidity_percent;
    uint32_t sensor_value;

    uint32_t secure_input_buffer;
    uint32_t extracted_features;
    uint32_t normalized_features;
    uint32_t anomaly_score;
    uint32_t secure_output_buffer;
    uint32_t signature;
    uint32_t inference_time_ms;

    uint32_t system_cycles;
    uint32_t total_inferences;
    uint32_t total_alerts;
    uint32_t validation_failures;
    uint32_t inference_failures;
    uint32_t dht_error_count;

    uint8_t access_allowed;
    uint8_t verified_alert;
    uint8_t blocked_attack;
    uint8_t node_health;
    uint8_t current_state;
    uint8_t dht_status;
} PrototypeStatus_t;

/* -------------------------------------------------------------------------- */
/* GLOBAL VARIABLES FOR DEBUG EXPRESSIONS                                     */
/* -------------------------------------------------------------------------- */

PrototypeStatus_t g_status;

uint32_t g_temperature_c = 0;
uint32_t g_humidity_percent = 0;
uint32_t g_sensor_value = 0;

uint32_t g_secure_input_buffer = 0;
uint32_t g_extracted_features = 0;
uint32_t g_normalized_features = 0;
uint32_t g_anomaly_score = 0;
uint32_t g_secure_output_buffer = 0;
uint32_t g_signature = 0;

uint32_t g_inference_start = 0;
uint32_t g_inference_end = 0;
uint32_t g_inference_time_ms = 0;

uint32_t g_system_cycles = 0;
uint32_t g_total_inferences = 0;
uint32_t g_total_alerts = 0;
uint32_t g_validation_failures = 0;
uint32_t g_inference_failures = 0;
uint32_t g_dht_error_count = 0;

uint8_t g_access_allowed = 0;
uint8_t g_verified_alert = 0;
uint8_t g_blocked_attack = 0;
uint8_t g_node_health = 1;
uint8_t g_dht_status = 0;

NodeState_t g_node_state = NODE_STATE_NORMAL;

/*
 * Test modes:
 * g_security_test_mode = 0: normal secure access allowed
 * g_security_test_mode = 1: simulated blocked access
 *
 * g_tamper_test_mode = 0: real DHT11 input
 * g_tamper_test_mode = 1: simulated tampered input
 */
uint8_t g_security_test_mode = 0;
uint8_t g_tamper_test_mode = 1;

/*
 * Demonstration threshold:
 * - 3 makes verified anomaly easier to demonstrate with real DHT11 readings.
 * - Increase later for stricter anomaly detection.
 */
uint32_t g_anomaly_threshold = 3;

/* -------------------------------------------------------------------------- */
/* MICROSECOND DELAY USING DWT                                                */
/* -------------------------------------------------------------------------- */

void DWT_Delay_Init(void)
{
    CoreDebug->DEMCR |= CoreDebug_DEMCR_TRCENA_Msk;
    DWT->CYCCNT = 0;
    DWT->CTRL |= DWT_CTRL_CYCCNTENA_Msk;
}

uint32_t micros(void)
{
    return DWT->CYCCNT / (SystemCoreClock / 1000000U);
}

void delay_us(uint32_t us)
{
    uint32_t start = micros();

    while ((micros() - start) < us)
    {
    }
}

/* ============================================================
 * NON-SECURE SIDE
 * Responsibilities:
 * - Read DHT11 sensor
 * - Provide sensor value to secure side
 * - Show LED result/status
 * ============================================================
 */

/* -------------------------------------------------------------------------- */
/* DHT11 LOW-LEVEL FUNCTIONS                                                  */
/* -------------------------------------------------------------------------- */

void DHT11_Set_Pin_Output(void)
{
    GPIO_InitTypeDef GPIO_InitStruct = {0};

    GPIO_InitStruct.Pin = DHT11_GPIO_PIN;
    GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_OD;
    GPIO_InitStruct.Pull = GPIO_PULLUP;
    GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

    HAL_GPIO_Init(DHT11_GPIO_PORT, &GPIO_InitStruct);
}

void DHT11_Set_Pin_Input(void)
{
    GPIO_InitTypeDef GPIO_InitStruct = {0};

    GPIO_InitStruct.Pin = DHT11_GPIO_PIN;
    GPIO_InitStruct.Mode = GPIO_MODE_INPUT;
    GPIO_InitStruct.Pull = GPIO_PULLUP;

    HAL_GPIO_Init(DHT11_GPIO_PORT, &GPIO_InitStruct);
}

uint8_t DHT11_Wait_For_State(GPIO_PinState state, uint32_t timeout_us)
{
    uint32_t start = micros();

    while (HAL_GPIO_ReadPin(DHT11_GPIO_PORT, DHT11_GPIO_PIN) != state)
    {
        if ((micros() - start) > timeout_us)
        {
            return 0;
        }
    }

    return 1;
}

/*
 * DHT11_Read return codes:
 * 0 = OK
 * 1 = response low timeout
 * 2 = response high timeout
 * 3 = first bit low timeout
 * 4 = bit start high timeout
 * 5 = bit high end timeout
 * 6 = checksum error
 */
uint8_t DHT11_Read(uint8_t *temperature, uint8_t *humidity)
{
    uint8_t data[5] = {0, 0, 0, 0, 0};

    DHT11_Set_Pin_Output();

    /* Idle high */
    HAL_GPIO_WritePin(DHT11_GPIO_PORT, DHT11_GPIO_PIN, GPIO_PIN_SET);
    HAL_Delay(2);

    /* Start signal: low for at least 18 ms */
    HAL_GPIO_WritePin(DHT11_GPIO_PORT, DHT11_GPIO_PIN, GPIO_PIN_RESET);
    HAL_Delay(20);

    /* Release line */
    HAL_GPIO_WritePin(DHT11_GPIO_PORT, DHT11_GPIO_PIN, GPIO_PIN_SET);
    delay_us(30);

    DHT11_Set_Pin_Input();

    /*
     * DHT11 response:
     * sensor pulls low ~80us, high ~80us, then starts data.
     * Relaxed 300us timeouts avoid firmware freezing.
     */
    if (!DHT11_Wait_For_State(GPIO_PIN_RESET, 300)) return 1;
    if (!DHT11_Wait_For_State(GPIO_PIN_SET,   300)) return 2;
    if (!DHT11_Wait_For_State(GPIO_PIN_RESET, 300)) return 3;

    for (uint8_t i = 0; i < 40; i++)
    {
        uint32_t high_start;
        uint32_t high_time;

        /* Wait for data high pulse */
        if (!DHT11_Wait_For_State(GPIO_PIN_SET, 300)) return 4;

        high_start = micros();

        /* Wait for high pulse to end */
        if (!DHT11_Wait_For_State(GPIO_PIN_RESET, 300)) return 5;

        high_time = micros() - high_start;

        data[i / 8] <<= 1;

        /*
         * ~26-28us high means 0.
         * ~70us high means 1.
         */
        if (high_time > 40)
        {
            data[i / 8] |= 1;
        }
    }

    if (((data[0] + data[1] + data[2] + data[3]) & 0xFF) != data[4])
    {
        return 6;
    }

    *humidity = data[0];
    *temperature = data[2];

    return 0;
}

uint32_t NonSecure_Read_DHT11_Sensor_Index(void)
{
    uint8_t temp = 0;
    uint8_t hum = 0;

    g_dht_status = DHT11_Read(&temp, &hum);

    if (g_dht_status == 0)
    {
        g_temperature_c = temp;
        g_humidity_percent = hum;

        /*
         * Combined environmental index.
         * Example: temp 25 + humidity 60 = 85
         */
        return (uint32_t)temp + (uint32_t)hum;
    }

    g_dht_error_count++;
    return 0;
}

void NonSecure_Normal_LED_Task(void)
{
    /* Normal monitoring indication: slow blink */
    HAL_GPIO_TogglePin(GPIOB, GPIO_PIN_7);
    HAL_Delay(500);
}

void NonSecure_VerifiedAlert_LED_Task(void)
{
    /* Verified anomaly indication: fast blink burst */
    for (int i = 0; i < 5; i++)
    {
        HAL_GPIO_TogglePin(GPIOB, GPIO_PIN_7);
        HAL_Delay(100);
    }
}

void NonSecure_Blocked_LED_Task(void)
{
    /* Error/blocked indication: one-second blink */
    HAL_GPIO_WritePin(GPIOB, GPIO_PIN_7, GPIO_PIN_SET);
    HAL_Delay(1000);
    HAL_GPIO_WritePin(GPIOB, GPIO_PIN_7, GPIO_PIN_RESET);
    HAL_Delay(1000);
}

/* ============================================================
 * SECURE SIDE
 * Responsibilities:
 * - Check access request
 * - Validate sensor input
 * - Process features
 * - Run TinyML-style inference
 * - Generate verified alert signature
 * ============================================================
 */

uint8_t Secure_Check_Access(uint8_t request_from_non_secure)
{
    if (request_from_non_secure == 1)
    {
        return 1;
    }

    return 0;
}

uint8_t Secure_Validate_Input(uint32_t input_value)
{
    /*
     * DHT11 combined temperature + humidity index is expected below 150.
     */
    if (input_value <= 150)
    {
        return 1;
    }

    return 0;
}

uint32_t Secure_Extract_Features(uint32_t sensor_value)
{
    return sensor_value / 2;
}

uint32_t Secure_Normalize_Features(uint32_t feature_value)
{
    return feature_value / 2;
}

uint8_t Secure_Validate_Features(uint32_t feature_value)
{
    if (feature_value <= 100)
    {
        return 1;
    }

    return 0;
}

uint32_t Secure_Run_TinyML_Inference(uint32_t feature_value)
{
    /*
     * Simulated TinyML scoring model.
     * Later this function will be replaced by real TinyML inference.
     */
    uint32_t weight = 3;
    uint32_t bias = 5;
    uint32_t score = (feature_value * weight) + bias;

    if (score > 55)
    {
        return score - 55;
    }

    return 0;
}

uint32_t Secure_Verify_Alert(uint32_t sensor_value, uint32_t anomaly_score)
{
    /*
     * Simulated HSE-style signature.
     */
    return (sensor_value * 31) ^ (anomaly_score * 17) ^ 0xA5A5;
}

/*
 * Secure callable wrapper simulation.
 * In the real TrustZone split, the Non-Secure side will call a secure gateway
 * function similar to this.
 */
NodeState_t Secure_Process_Request(uint32_t sensor_value)
{
    g_secure_input_buffer = sensor_value;

    if (Secure_Validate_Input(g_secure_input_buffer) == 0)
    {
        g_validation_failures++;
        g_blocked_attack = 1;
        g_verified_alert = 0;
        return NODE_STATE_TAMPER_BLOCKED;
    }

    g_inference_start = HAL_GetTick();

    g_extracted_features = Secure_Extract_Features(g_secure_input_buffer);
    g_normalized_features = Secure_Normalize_Features(g_extracted_features);

    if (Secure_Validate_Features(g_normalized_features) == 0)
    {
        g_validation_failures++;
        g_blocked_attack = 1;
        g_verified_alert = 0;
        return NODE_STATE_FEATURE_BLOCKED;
    }

    g_anomaly_score = Secure_Run_TinyML_Inference(g_normalized_features);

    if (g_anomaly_score == 0 && g_normalized_features > 80)
    {
        g_inference_failures++;
    }

    g_secure_output_buffer = g_anomaly_score;

    g_inference_end = HAL_GetTick();
    g_inference_time_ms = g_inference_end - g_inference_start;

    g_total_inferences++;

    if (g_anomaly_score > g_anomaly_threshold)
    {
        g_signature = Secure_Verify_Alert(sensor_value, g_anomaly_score);

        if (g_signature != 0)
        {
            g_total_alerts++;
            g_verified_alert = 1;
            g_blocked_attack = 0;
            return NODE_STATE_VERIFIED_ALERT;
        }

        return NODE_STATE_ANOMALY;
    }

    g_verified_alert = 0;
    g_blocked_attack = 0;
    return NODE_STATE_NORMAL;
}

/* -------------------------------------------------------------------------- */
/* STATUS UPDATE                                                              */
/* -------------------------------------------------------------------------- */

void Update_Status(void)
{
    g_status.temperature_c = g_temperature_c;
    g_status.humidity_percent = g_humidity_percent;
    g_status.sensor_value = g_sensor_value;
    g_status.secure_input_buffer = g_secure_input_buffer;
    g_status.extracted_features = g_extracted_features;
    g_status.normalized_features = g_normalized_features;
    g_status.anomaly_score = g_anomaly_score;
    g_status.secure_output_buffer = g_secure_output_buffer;
    g_status.signature = g_signature;
    g_status.inference_time_ms = g_inference_time_ms;

    g_status.system_cycles = g_system_cycles;
    g_status.total_inferences = g_total_inferences;
    g_status.total_alerts = g_total_alerts;
    g_status.validation_failures = g_validation_failures;
    g_status.inference_failures = g_inference_failures;
    g_status.dht_error_count = g_dht_error_count;

    g_status.access_allowed = g_access_allowed;
    g_status.verified_alert = g_verified_alert;
    g_status.blocked_attack = g_blocked_attack;
    g_status.node_health = g_node_health;
    g_status.current_state = (uint8_t)g_node_state;
    g_status.dht_status = g_dht_status;
}

/* -------------------------------------------------------------------------- */
/* MAIN                                                                       */
/* -------------------------------------------------------------------------- */

int main(void)
{
  HAL_Init();

  SystemPower_Config();
  SystemClock_Config();

  MX_GPIO_Init();
  DWT_Delay_Init();

  MX_USART3_UART_Init();
  MX_ICACHE_Init();
  MX_USB_OTG_HS_PCD_Init();

  while (1)
  {
      g_system_cycles++;
      g_node_health = 1;
      g_node_state = NODE_STATE_NORMAL;

      g_verified_alert = 0;
      g_blocked_attack = 0;

      /*
       * Non-Secure side requests access to Secure processing.
       */
      g_access_allowed = Secure_Check_Access(g_security_test_mode == 0 ? 1 : 0);

      if (g_access_allowed == 1)
      {
          /*
           * Non-Secure side reads sensor or simulates tamper.
           */
          if (g_tamper_test_mode == 1)
          {
              g_sensor_value = 999;
          }
          else
          {
              g_sensor_value = NonSecure_Read_DHT11_Sensor_Index();
          }

          /*
           * DHT11 sensor read failed.
           */
          if (g_tamper_test_mode == 0 && g_dht_status != 0)
          {
              g_blocked_attack = 1;
              g_verified_alert = 0;
              g_node_state = NODE_STATE_SENSOR_ERROR;

              Update_Status();
              NonSecure_Blocked_LED_Task();
              continue;
          }

          /*
           * Non-Secure side sends sensor value to simulated Secure callable wrapper.
           */
          g_node_state = Secure_Process_Request(g_sensor_value);

          Update_Status();

          if (g_node_state == NODE_STATE_VERIFIED_ALERT)
          {
              NonSecure_VerifiedAlert_LED_Task();
          }
          else if (g_node_state == NODE_STATE_NORMAL)
          {
              NonSecure_Normal_LED_Task();
          }
          else
          {
              NonSecure_Blocked_LED_Task();
          }
      }
      else
      {
          /*
           * Secure access denied.
           */
          g_blocked_attack = 1;
          g_verified_alert = 0;
          g_node_state = NODE_STATE_ACCESS_BLOCKED;

          Update_Status();
          NonSecure_Blocked_LED_Task();
      }

      /*
       * DHT11 should not be read too frequently.
       */
      HAL_Delay(1500);
  }
}

/* -------------------------------------------------------------------------- */
/* STM32 INITIALIZATION FUNCTIONS                                             */
/* -------------------------------------------------------------------------- */

void SystemClock_Config(void)
{
  RCC_OscInitTypeDef RCC_OscInitStruct = {0};
  RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

  if (HAL_PWREx_ControlVoltageScaling(PWR_REGULATOR_VOLTAGE_SCALE2) != HAL_OK)
  {
    Error_Handler();
  }

  RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI | RCC_OSCILLATORTYPE_HSE;
  RCC_OscInitStruct.HSEState = RCC_HSE_BYPASS;
  RCC_OscInitStruct.HSIState = RCC_HSI_ON;
  RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;
  RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON;
  RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSE;
  RCC_OscInitStruct.PLL.PLLMBOOST = RCC_PLLMBOOST_DIV1;
  RCC_OscInitStruct.PLL.PLLM = 1;
  RCC_OscInitStruct.PLL.PLLN = 16;
  RCC_OscInitStruct.PLL.PLLP = 8;
  RCC_OscInitStruct.PLL.PLLQ = 2;
  RCC_OscInitStruct.PLL.PLLR = 2;
  RCC_OscInitStruct.PLL.PLLRGE = RCC_PLLVCIRANGE_1;
  RCC_OscInitStruct.PLL.PLLFRACN = 0;

  if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK)
  {
    Error_Handler();
  }

  RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK | RCC_CLOCKTYPE_SYSCLK
                              | RCC_CLOCKTYPE_PCLK1 | RCC_CLOCKTYPE_PCLK2
                              | RCC_CLOCKTYPE_PCLK3;
  RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_HSI;
  RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;
  RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV1;
  RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV1;
  RCC_ClkInitStruct.APB3CLKDivider = RCC_HCLK_DIV1;

  if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_0) != HAL_OK)
  {
    Error_Handler();
  }
}

static void SystemPower_Config(void)
{
  HAL_PWREx_DisableUCPDDeadBattery();

  if (HAL_PWREx_ConfigSupply(PWR_SMPS_SUPPLY) != HAL_OK)
  {
    Error_Handler();
  }
}

static void MX_ICACHE_Init(void)
{
  if (HAL_ICACHE_ConfigAssociativityMode(ICACHE_1WAY) != HAL_OK)
  {
    Error_Handler();
  }

  if (HAL_ICACHE_Enable() != HAL_OK)
  {
    Error_Handler();
  }
}

static void MX_USB_OTG_HS_PCD_Init(void)
{
  hpcd_USB_OTG_HS.Instance = USB_OTG_HS;
  hpcd_USB_OTG_HS.Init.dev_endpoints = 9;
  hpcd_USB_OTG_HS.Init.speed = PCD_SPEED_HIGH;
  hpcd_USB_OTG_HS.Init.phy_itface = USB_OTG_HS_EMBEDDED_PHY;
  hpcd_USB_OTG_HS.Init.Sof_enable = DISABLE;
  hpcd_USB_OTG_HS.Init.low_power_enable = DISABLE;
  hpcd_USB_OTG_HS.Init.lpm_enable = DISABLE;
  hpcd_USB_OTG_HS.Init.use_dedicated_ep1 = DISABLE;
  hpcd_USB_OTG_HS.Init.vbus_sensing_enable = DISABLE;
  hpcd_USB_OTG_HS.Init.dma_enable = DISABLE;

  if (HAL_PCD_Init(&hpcd_USB_OTG_HS) != HAL_OK)
  {
    Error_Handler();
  }
}

static void MX_GPIO_Init(void)
{
  GPIO_InitTypeDef GPIO_InitStruct = {0};

  __HAL_RCC_GPIOB_CLK_ENABLE();
  __HAL_RCC_GPIOC_CLK_ENABLE();
  __HAL_RCC_GPIOH_CLK_ENABLE();
  __HAL_RCC_GPIOA_CLK_ENABLE();
  __HAL_RCC_GPIOF_CLK_ENABLE();

  /* LD2 Blue = PB7 */
  GPIO_InitStruct.Pin = GPIO_PIN_7;
  GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;
  GPIO_InitStruct.Pull = GPIO_NOPULL;
  GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;
  HAL_GPIO_Init(GPIOB, &GPIO_InitStruct);

  /* DHT11 data pin = Arduino D7 = PF13 */
  GPIO_InitStruct.Pin = DHT11_GPIO_PIN;
  GPIO_InitStruct.Mode = GPIO_MODE_INPUT;
  GPIO_InitStruct.Pull = GPIO_PULLUP;
  HAL_GPIO_Init(DHT11_GPIO_PORT, &GPIO_InitStruct);

  HAL_GPIO_WritePin(UCPD_DBn_GPIO_Port, UCPD_DBn_Pin, GPIO_PIN_RESET);

  GPIO_InitStruct.Pin = VBUS_SENSE_Pin;
  GPIO_InitStruct.Mode = GPIO_MODE_ANALOG;
  GPIO_InitStruct.Pull = GPIO_NOPULL;
  HAL_GPIO_Init(VBUS_SENSE_GPIO_Port, &GPIO_InitStruct);

  GPIO_InitStruct.Pin = UCPD_FLT_Pin;
  GPIO_InitStruct.Mode = GPIO_MODE_INPUT;
  GPIO_InitStruct.Pull = GPIO_NOPULL;
  HAL_GPIO_Init(UCPD_FLT_GPIO_Port, &GPIO_InitStruct);

  GPIO_InitStruct.Pin = GPIO_PIN_15;
  GPIO_InitStruct.Mode = GPIO_MODE_ANALOG;
  GPIO_InitStruct.Pull = GPIO_NOPULL;
  HAL_GPIO_Init(GPIOB, &GPIO_InitStruct);

  GPIO_InitStruct.Pin = GPIO_PIN_15;
  GPIO_InitStruct.Mode = GPIO_MODE_ANALOG;
  GPIO_InitStruct.Pull = GPIO_NOPULL;
  HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

  GPIO_InitStruct.Pin = UCPD_DBn_Pin;
  GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;
  GPIO_InitStruct.Pull = GPIO_NOPULL;
  GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;
  HAL_GPIO_Init(UCPD_DBn_GPIO_Port, &GPIO_InitStruct);
}

static void MX_USART3_UART_Init(void)
{
  huart3.Instance = USART3;
  huart3.Init.BaudRate = 115200;
  huart3.Init.WordLength = UART_WORDLENGTH_8B;
  huart3.Init.StopBits = UART_STOPBITS_1;
  huart3.Init.Parity = UART_PARITY_NONE;
  huart3.Init.Mode = UART_MODE_TX_RX;
  huart3.Init.HwFlowCtl = UART_HWCONTROL_NONE;
  huart3.Init.OverSampling = UART_OVERSAMPLING_16;
  huart3.Init.OneBitSampling = UART_ONE_BIT_SAMPLE_DISABLE;
  huart3.AdvancedInit.AdvFeatureInit = UART_ADVFEATURE_NO_INIT;

  if (HAL_UART_Init(&huart3) != HAL_OK)
  {
    Error_Handler();
  }
}

int __io_putchar(int ch)
{
    HAL_UART_Transmit(&huart3, (uint8_t *)&ch, 1, 10);
    return ch;
}

void Error_Handler(void)
{
  while (1)
  {
    HAL_GPIO_TogglePin(GPIOB, GPIO_PIN_7);
    HAL_Delay(100);
  }
}

#ifdef USE_FULL_ASSERT
void assert_failed(uint8_t *file, uint32_t line)
{
}
#endif /* USE_FULL_ASSERT */